Name:_________________________________

Solve the following problems in the space provided.  Use the back of the page if needed.  Each problem is worth 20 points.  You must show your work in a logical fashion starting with the correctly applied physical principles which are on the last page.  Your score will be maximized if your work is easy to follow because partial credit will be awarded.

1.  A long straight wire carries a current of 10.0A upward while a proton is heading to the right at 3000m/s.  (a)Find the strength of the magnetic field felt by the proton.  (b)Indicate the direction of this field in the drawing.  (c)Find the magnitude of the force on the proton and (d)indicate the direction of this force in the drawing.

(a)The field due to a wire is .

Putting in the numbers,

(b)Using the right hand rule, the field is into the page.

(c)The force on a moving charge is .

The angle between the velocity and field is 90˚.  So,

(b)Again using the right hand rule, the force is upward.

2. A horizontal loop of wire of radius 3.00cm has a resistance of 1.00Ω.  There is an upward magnetic field through the loop.  There is no power supply to the loop, yet there is a current in the loop of 2.50mA in the direction shown.  Is the magnetic field increasing, decreasing or staying the same?  If the field is changing, find the rate of change in Tesla per second.

According to Faraday's Law the changing flux through the loop creates the current.  Therefor, the field must be changing.  According to Lenz's Rule the current will create a magnetic field that opposes the change in flux.  The field due to the current is upward in the loop and therefor the change in field is downward.  If the changing field is downward, that means that the field must be decreasing.

The rate of decrease is determined from Faraday's Law, .

Using the definition of flux, .

Substituting into Faraday's Law, .

Using Ohm's Rule, .

Plugging in the numbers,

3.  A coiling is spinning in a constant magnetic field.  The output voltage of the coil versus time is shown in the sketch on the left.  Sketch the output voltage versus time when the rate the coils spins is doubled.  You must explain your reasoning using the appropriate physical concepts for full credit.

Again, Faraday's Law is the key idea.  By spinning the coils at twice the rate, the rate at which the flux lines are cut is doubled.  Therefor, the peak voltage will be doubled.  This can be seen from the generator equation  which was derived from Faraday's Law.

4.  A 4.00µF capacitor is connected directly to an AC power supply that produces an rms output of 12.0V.  The resulting rms current is 20.0mA.  Find the frequency of the source.

The impedance of the capacitor is, .

The rms current is related to the rms voltage by the definition of impedance,

.

The linear frequency is related to the angular frequency according to,

5. A 5.00mW helium-neon laser produces a beam with a radius of 0.500mm at a wavelength of 632.8nm.  Find (a)the frequency of this light, (b)the peak electric field in the beam and (c)the peak magnetic field in the beam.

(a)The frequency is related to the wavelength according to, .

(b)The intensity of the beam is .

From the definition of the Poynting Vector, , and the ratio of the fields, , the peak fields can be found.

(c)The magnetic field can be found from the electric field,

Laws, Principles, Useful Relationships, and Other Information

Coulomb's Rule 	Def'n of E-Field 	E-Field Due to Point Charge

Potential Energy 	Def'n of Electric Potential

Potential from E-field 	Potential Due to Point Charge

Def'n of Capacitance 	Capacitance of Parallel Plates 	Cap's in Series

Cap's in Parallel 	Energy in C's 	Energy in E-Fields

Def'n of Current 	Def'n of Resistance 	Ohm's Rule

Electric Power 	Resistors in Series 	Resistors in Parallel

RC Charge 	RC Discharge

Biot-Savart Rule 	B-Field of a Wire 	Def'n of B-Field

Force on a Charge 	Torque on Current Loop 	Def'n  of Flux

Faraday's Law  	Generator Output

The Def'n of Inductance 	LR "charging" 	LR "discharging"

Energy in Inductors 	Energy Density in a B-Field

Transformers:  and 	peak-to-rms:

Def'n of Impedance 	Impedances:

RLC Circuit: Impedance      Phase     Resonance Freq.

EM Waves:           	Def'n of Poynting Vector

Physical Constants

c = 3.00x108m/s

me = 9.11x10-31kg = 0.511MeV/c2	q= -e = -1.60x10-19C

mp = 1.67x10-27kg = 940MeV/c2	q= e = +1.60x10-19C

mn = 1.67x10-27kg = 938MeV/c2	q= 0